CONDITIONS

FOR HOLDING A TENDER FOR THE SUPPLY OF LIQUEFIED NATURAL GAS (LNG)

FOR THE NEEDS OF BULGARGAZ PLC FOR MARCH 2024

- SUBJECT OF THE TENDER: Supply of liquefied natural gas (LNG) for the needs of Bulgargaz PLC for March 2024.

- TERMS OF DELIVERY:

- Total quantity: 1 000 000 MWh

  - Delivery Profile:

| Month | MWh | Operational tolerance |
| --- | --- | --- |
| March 2024 | 1,000,000 | +/- 5 % |

  - Minimum/maximum deviation of the delivered quantity from the requested quantity – to be declared but not more than +/- 5%.

  - Delivery window

27th March 2024 (36 hours)

- LNG Quality:

The peak calorific value of each standard cubic metre of gas (on a dry gas basis) shall be not less than 8750 kcal (eight thousand seven hundred and fifty kcal) and not more than 10 427 kcal (ten thousand four hundred and twenty-seven kcal).

The components must be between the following lower and upper limits (as mole percentages) [Reference conditions: 150C/150C/ideal gas]:

| Chemical composition | Min. % | Max. % |
| --- | --- | --- |
| Methane CH 4 | 84.79 % | 100.00 % |
| Ethan C H 26 | 0.00 % | 10.00 % |
| Propane C H 38 | 0.00 % | 4.00 % |
| iC H 410 | 0.00 % | 1.00 % |
| nC H 410 | 0.00 % | 1.50 % |
| C H 512 | 0.00 % | 0.23 % |
| N 2 | 0.00 % | 1.40 % |
| Gross calorific value | 8,900 Kcal/Sm 3 | 10,427 Kcal/Sm 3 |
| Wobbe index | 47,30 Mj/m 3 | 53.95 Mj/m 3 |
| H S 2 | - | 5.0 mg/Nm 3 |
| Mercaptan Sulphur | - | 5.0 mg/Nm 3 |
| Total sulphur, including mercaptans | - | 50 mg/Nm 3 |

Hydrocarbon dew point: should not be higher than -5°C in the range of 1 to 80 barg.

Water dew point: must not be higher than -80 C at 80 barg. pressure

LNG shall not cover particles of a size that does not pass through a 0.0098-inch sieve.

LNG does not include water, oxygen, and carbon dioxide.

There shall be no active bacteria or bacteria coating materials in the LNG, including but not limited to bacteria that reduce sulphates or produce acid.

LNG should be free of poisonous and hazardous materials.

- Delivery period: March 2024

- Place of delivery (unloading):

LNG terminal Revithoussa, Greece

- Delivery Terms - DES (Delivery Ex-Ship)

- Seller's liability

In the event Seller fails to deliver the quantity or any portion thereof, unless due to Force Majeure, Seller shall pay Buyer an amount equal to (i) if Buyer fails to purchase replacement gas, Buyer's costs, associated with the failure or termination of the resale arrangements with respect to Seller's undelivered quantity, including Buyer's respective facility costs, capacity reservations, transportation, reasonable marketing costs and other costs incurred by Buyer, if any, as a result of Seller's failure to provide; or (ii) if Buyer purchases replacement LNG or gas to replace Seller's undelivered quantity, the amount by which the price paid by Buyer for the replacement gas exceeds the Contract Sales Price multiplied by Seller's undelivered quantity, plus additional transportation and logistics costs if any, plus other reasonable costs incurred by Buyer if any, less any cost savings (including transportation costs) realized by Buyer.

- Demurrage - The demurrage rate on a daily and pro-rata basis is the rate published under the heading Atlantic "Shipping Rate" in Platts LNG Daily on the relevant scheduled delivery date. If the Platts rate is not published on the applicable scheduled delivery date, then the demurrage rate shall be the last published Platts Atlantic Rate immediately applicable to the scheduled delivery date. Seller shall have the obligation to provide evidence of the calculated demurrage rate.

- REQUIREMENTS FOR PARTICIPANTS:

- Sources of LNG supply - the source must be from countries without sanctions, embargoes or any trade restrictions;

- Ownership of the company - the ownership (ultimate beneficiary) must be clear, reputable and free of ties to countries subject to sanctions, embargoes or any trade restrictions;

- Credibility with banks and reputation - the supplier company must be acceptable to banks and credible for financing the agreed supplies;

- To have experience in LNG supply - total LNG supply of at least the equivalent of 2 000 000 MWh ≈ 6 800 000 MMBtu/year for the last three years;

- Not to be in liquidation or bankruptcy proceedings and to not be declared bankrupt under the national law of registration.

- STAGES OF THE PROCEDURE

Stage I. Pre-selection of candidates

By 22:00, local time, until 22.01.2024, all interested companies who wish to participate in this tender procedure shall submit to Bulgargaz PLC by e-mail to the following address: tenders@bulgargaz.bg a letter of intent indicating their interest in further participation in the procedure and attaching a completed Bulgargaz PLC Questionnaire for getting to know a potential partner company, as per the template - Annex No. 1 to these Terms and Conditions and as well as a standard MSPA;

In case the applicant has participated in previous tender procedures of Bulgargaz PLC for the supply of LNG, has been admitted to the stage of submission of a binding offer, and there is no change in the circumstances declared in the Questionnaire, it is not necessary to submit a completed Questionnaire.

A committee, appointed by order of the Executive Director of Bulgargaz PLC, will verify the compliance of the applicants with the requirements for participation in the tender procedure, based on the submitted documents and will notify the applicants admitted participating in the next stage of the tender procedure by 16:00, local time, on 23.01.2024, by e-mail to the e-mail addresses they have provided.

No applicant will be admitted to Phase II of the tender procedure:

- who has submitted a letter of intent outside the time limit;

- who has not submitted a completed Questionnaire to Bulgargaz PLC to get to know a potential partner company according to the template - Annex 1;

- in the questionnaire of which there is insufficient information, or the information indicates that he does not meet the requirements for participation in the procedure.

By 16:00, local time, on 23.01.2024 Bulgargaz PLC will inform the candidates not admitted participating in Stage II of the tender procedure with an individual motivated letter by e-mail, to the e-mail addresses indicated by them.

Stage II. Submission of binding offers

1. Deadline for submission: 22:00 local time, 25.01.2024.

Candidates admitted to participating in Stage II of the tender procedure shall submit an electronically signed Offer in the form - Annex No. 2 to these Conditions, together with the annexes thereto, by e-mail to the following address: tenders@bulgargaz.bg, in an archived format with a password by 22:00 local time on 25.01.2024.

The password must be sent by a separate email to the same email address no earlier than 9:00 a.m. and no later than 11:00 a.m., local time, on 26.01.2023.

2. Mandatory content requirements:

  - LNG quantity

- the quantity offered per month shall not be less than 1 000 000 MWh or 1 cargo;

- minimum/maximum deviation of the delivered quantity from the requested quantity - not more than +/- 5%;

  - Quality of LNG

The peak calorific value of each standard cubic metre of gas (on a dry gas basis) shall be not less than 8750 kcal (eight thousand seven hundred and fifty kcal) and not more than 10 427 kcal (ten thousand four hundred and twenty-seven kcal).

The components should be between the following lower and upper limits (as mole percentages) [Reference conditions: 150C/150C/ideal gas]:

| Chemical composition | Min. % | Max. % |
| --- | --- | --- |
| Methane CH 4 | 84.79 % | 100.00 % |
| Ethan C H 26 | 0.00 % | 10.00 % |
| Propane C H 38 | 0.00 % | 4.00 % |
| iC H 410 | 0.00 % | 1.00 % |
| nC H 410 | 0.00 % | 1.50 % |
| C H 512 | 0.00 % | 0.23 % |
| N 2 | 0.00 % | 1.40 % |
| Gross calorific value | 8,900 Kcal/Sm 3 | 10,427 Kcal/Sm 3 |
| Wobbe index | 47,30 Mj/m 3 | 53.95 Mj/m 3 |
| H S 2 | - | 5.0 mg/Nm 3 |
| Mercaptan Sulphur | - | 5.0 mg/Nm 3 |
| Total sulphur, including mercaptans | - | 50 mg/Nm 3 |

Hydrocarbon dew point: should not be higher than -5°C in the range of 1 to 80 barg.

Water dew point: must not be higher than -80 C at 80 barg. pressure

LNG shall not cover particles of a size that does not pass through a 0.0098-inch sieve.

LNG does not include water, oxygen and carbon dioxide.

There shall be no active bacteria or bacteria coating materials in the LNG, including but not limited to bacteria that reduce sulphates or produce acid.

LNG should be free of poisonous and hazardous materials.

  - Terms of Delivery: DES (Delivery Ex-Ship)

LNG terminal Revithoussa, Greece

  - Price:

the offer price should reference the TTF front month for the month in which the delivery window falls with an indicated discount in Euro per MWh and include all delivery costs to the LNG terminal in Revithoussa, Greece

TTFfm = the value of the TTF Front Month Index published in Argus European Natural Gas - Daily Natural Gas Market Prices, in the column headed "Price", for the TTF (Title Transfer Facility) for the last business day of the month preceding the month of delivery. The TTF fm is the arithmetic mean between the 'Buy' and 'Sell' values for each day of the month for which the corresponding month is the following month.

  - Delivery window

27th March 2024 (36 hours)

  - Method and terms of payment upon conclusion of a contract:

the amount of the advance payment: may not exceed 50%;

advance payment deadline: not earlier than 15 days before the delivery date;

number of days for payment after the last day of delivery: not less than 10 days.

  - Additional costs that may arise for Bulgargaz PLC during delivery (assumed boil-off - no more than 0.15% per day, other);

  - Declaration of absence of circumstances, in the form - Annex № 3 to these conditions.

  - Power of attorney - to be attached if the offer is signed by an attorney.

No participant will be admitted for evaluation and ranking:

- who has submitted an Offer and/or password outside the time limit;

- who has submitted an Offer the content of which does not comply with the above mandatory requirements or

- who has submitted a password that does not open the Offer.

Stage III. Evaluation and ranking of Offers

The Offers of the participants will be opened by the committee appointed by the order of the Executive Director of Bulgargaz PLC at 12:00 local time on 26.01.2023.

No participant will be admitted for evaluation and ranking:

- who has submitted an Offer and/or password outside the time limit;

- who has submitted an Offer the content of which does not comply with the above-mentioned mandatory requirements or a password that does not open the Offer submitted.

The Offers will be evaluated and ranked by the Commission according to the Methodology for Determining the Overall Evaluation of Tenders referenced to the TTF Index - Annex 4 to these Conditions.

By 17:00 local time on 26.01.2024 Bulgargaz PLC will notify by e-mail, to the e-mail addresses they have specified, the qualified participants with whom it will conclude contracts, up to the amount of the quantity announced for delivery in March 2024.

Within the same timeframe, Bulgargaz PLC will inform the participants not admitted to evaluation and ranking with an individual motivated letter by e-mail, to the e-mail addresses indicated by them.

Bulgargaz PLC reserves the right to terminate the procedure at any time without selecting a contractor.